IGC Mass Spectrometry Facility

Integrated Multiomics Sample Preparation

Biphasic Extraction from Cell Pellets (MTBE / MeOH)

University of Edinburgh  |  Institute of Genetics and Cancer  |  April 2026

A liquid/liquid biphasic extraction system is used to simultaneously recover and fractionate polar and non-polar compounds from the same sample, leaving a protein pellet suitable for proteomics analysis. This single workflow enables proteins, lipids, and polar metabolites to be recovered from a single cell pellet — critical for precious or limited samples.

NOTE This is the integrated multiomics extraction protocol. It is suitable for 1–2 million cells per sample. All solvents must be LC-MS grade. Contact the facility if you need guidance on sourcing.

1.  Overview of Fractions

| Reagent / Item | Detail / Specification |
| --- | --- |
| Upper phase (MTBE) | Non-polar — Lipids → Lipidomics analysis (Positive and Negative ion modes) |
| Lower phase (MeOH:H₂O) | Polar — Metabolites → pHILIC metabolomics method |
| Protein pellet | Proteins → Proteomics (PAC or standard workflow) |

2.  Reagents and Equipment

| Reagent / Item | Detail / Specification |
| --- | --- |
| Methanol (MeOH) | LC-MS grade |
| Methyl tert-butyl ether (MTBE) | LC-MS grade |
| Water (H₂O) | LC-MS grade / ultrapure |
| Butanol (BuOH) | LC-MS grade (for lipid reconstitution) |
| Internal standards (IS) | Optional — add to MeOH before step 1 if used; discuss with facility |
| Thermomixer or sonicator | 4°C capable; 2000 rpm |
| Microcentrifuge | Max speed (~16,100 x g), 4°C |
| SpeedVac or N₂ stream | For drying lipid fraction |
| 96-well plate or glass vials | For final LC-MS submission |

3.  Extraction Protocol

Scale: 1–2 million cells per ml extraction solvent. Cell pellets should be stored at −80°C before extraction. Perform all steps on ice or at 4°C.

- Add 100 μl MeOHEnsure the pellet is fully resuspended by pipetting or brief vortex before shaking.  (containing internal standard if used) to the cell pellet. Shake at 2000 rpm for 20 min at 4°C, or sonicate in an ultrasound bath for 5 min.

- Add 300 μl MTBE. Shake at 2000 rpm for 20 min at 4°C.

- Add 100 μl H₂O. Shake for 1 minute or vortex to induce phase separation.

- Centrifuge at maximum speed (~16,100 x g) for 10 minutes at 4°C.

Upper phase — Lipids (MTBE layer)

- Carefully collect the upper phase (MTBE, containing lipids) without disturbing the interphase or lower phase. Transfer to a clean tube.

- Dry under SpeedVac or a gentle N₂ stream.

- Reconstitute in 100 μl MeOH:BuOH (1:1).

- Place at −80°C for 10 min, then centrifuge at max speed for 10 min at 4°C.

- Transfer supernatant to a 96-well plate or glass vials for LC-MS submission.

Lower phase — Polar Metabolites (MeOH:H₂O layer)

- Centrifuge the lower phase at max speed for 10 min at 4°C.

- Transfer the supernatant to a clean tube. Place at −80°C for 10 min.

- Centrifuge again at max speed for 10 min at 4°C.

- Transfer supernatant to a 96-well plate or glass vials for LC-MS submission.

Protein Pellet

- Retain the protein pellet after both liquid phases have been removed. Store at −80°C.

- The pellet can be used for proteomics (PAC workflow — see Proteomics Sample Preparation Guide) and for total protein quantification by BCA for normalization.

CRITICAL Collect the upper (MTBE) phase first and carefully — any cross-contamination with the lower phase will introduce polar metabolites into the lipid fraction. Do not allow samples to warm above 4°C during the extraction. Enzymatic activity and metabolite degradation are significant at room temperature. All solvents must be LC-MS grade. Trace impurities from lower-grade solvents will appear as peaks in the data. Minor MTBE carry-over into the polar fraction is expected and does not affect pHILIC analysis.

TIP Internal standards (IS) should be added to the MeOH in step 1 so they are present throughout the extraction and partition with the appropriate fraction. Label tubes clearly before starting — upper and lower phases can look similar and are easily confused after centrifugation.

4.  LC-MS Analysis Methods

| Reagent / Item | Detail / Specification |
| --- | --- |
| Polar phase (metabolites) | pHILIC chromatography method |
| Non-polar phase (lipids) — positive mode | Lipids Positive method |
| Non-polar phase (lipids) — negative mode | Lipids Negative method |

NOTE Discuss analysis method selection with the facility at the point of sample submission. Both positive and negative ion mode lipidomics are typically run from the same lipid extract.