IGC Mass Spectrometry Facility

Lipidomics Sample Preparation

Phosphatidylinositol Phosphate (PIP) Extraction from Cell Pellets

University of Edinburgh  |  Institute of Genetics and Cancer  |  April 2026

Phosphatidylinositol phosphates (PIPs) are low-abundance, highly charged signalling lipids that require dedicated extraction chemistry for adequate recovery. Neutral lipids are first removed by a MeOH:CHCl₃ pre-extraction, then PIPs are partitioned into an MTBE/MeOH/HCl organic phase. The recovered PIPs are methylated with TMSD (trimethylsilyldiazomethane) to improve chromatographic behaviour and ionisation efficiency before LC-MS analysis.

1.  Reagents and Equipment

| Reagent / Item | Detail / Specification |
| --- | --- |
| Methanol (MeOH) | LC-MS grade |
| Chloroform (CHCl₃) | HPLC grade |
| Methyl tert-butyl ether (MTBE) | LC-MS grade |
| Water (H₂O) | LC-MS grade / ultrapure |
| Hydrochloric acid (HCl), 2 N | For biphasic partitioning |
| Hydrochloric acid (HCl), 0.1 N | For phase separation |
| TMSD (trimethylsilyldiazomethane), 2 M | In hexane; for PIP methylation |
| Glacial acetic acid | For quenching TMSD reaction |
| Internal standards (IS) | Optional — e.g. deuterated PIP standards; add to MeOH in step 1 |
| Sonicator (bath or probe) | For cell lysis and lipid extraction |
| Microcentrifuge | Max speed (~16,100 × g), 4°C |
| SpeedVac or N₂ stream | For drying |
| Glass tubes with Teflon-lined caps | CRITICAL — chloroform dissolves plastic |
| 96-well plate or glass vials | For final LC-MS submission |

NOTE Scale: 1–2 million cells per ml minimum. Higher cell input (up to 5 million) can be used for broader PIP coverage, as these are very low-abundance species. All solvents must be LC-MS grade. Perform all steps at 4°C or on ice. Use glassware throughout — chloroform and MTBE dissolve polypropylene.

2.  Extraction Protocol

Step 1 — Pre-extraction of Neutral Lipids

Neutral lipids (e.g. triglycerides, cholesteryl esters) are removed first to prevent them from dominating the downstream PIP fraction.

Add 1 ml ice-cold MeOH:CHCl₃ (2:1) to the cell pellet.

Sonicate for 10 minutes in an ultrasound bath (on ice).

Centrifuge at max speed for 10 minutes at 4°C.

Remove and discard the supernatant (this contains neutral lipids).

CRITICAL Do not skip the neutral lipid pre-extraction. PIPs are present at very low abundance relative to neutral lipids. Without this step, the PIP signal will be suppressed by the overwhelming neutral lipid background. Note: some PI and monophosphorylated PIPs (e.g. PI3P) may be partially lost in this step. If these are primary targets, consider using the direct acid lysis route instead.

Step 2 — PIP Extraction (Biphasic Partitioning)

The pre-extracted pellet is resuspended in an acidified MTBE/MeOH system that partitions PIPs into the upper organic phase.

Resuspend the pellet in 1000 µl MTBE/MeOH/2 N HCl (24:74:2, v/v/v).

Add 250 µl of 0.1 N HCl to induce phase separation.

Centrifuge at max speed for 10 minutes at 4°C.

Carefully collect the upper organic layer (containing PIPs). Transfer to a clean glass tube.

TIP The HCl in the extraction solvent protonates the phosphoinositide headgroups, making them less polar and improving partitioning into the organic phase. Do not substitute water for the HCl component — recovery of multiply phosphorylated PIPs (PIP₂, PIP₃) will be severely reduced.

Alternative: Direct Acid Lysis (No Pre-extraction)

If neutral lipid removal is not required (e.g. when only PIP species are of interest and ion suppression is acceptable):

Add 500 µl of 2 M HCl directly to the cell pellet.

Sonicate for 10 minutes (on ice).

Centrifuge at max speed for 10 minutes at 4°C. Remove and discard the HCl supernatant.

Resuspend the pellet in 1000 µl MTBE/MeOH/2 N HCl (24:74:0.2, v/v/v).

Add 250 µl of 0.1 N HCl. Centrifuge at max speed for 10 minutes at 4°C.

Collect the upper organic layer for TMSD derivatisation.

NOTE The acid lysis variant uses a lower HCl proportion in the biphasic step (0.2 vs 2) because the pellet has already been acid-treated. Both routes converge at the TMSD derivatisation step below. Caution: 2 M HCl can partially dephosphorylate PIP₂ and PIP₃. Work quickly and keep samples on ice throughout to minimise hydrolysis.

3.  TMSD Methylation (Derivatisation)

Methylation of the phosphate groups on PIPs improves their chromatographic behaviour and ionisation efficiency for LC-MS detection.

To the collected MTBE phase containing PIPs, add 50 µl of 2 M TMSD (in hexane).

Incubate at room temperature for 10 minutes. Open the tube briefly after 30 seconds to release diazomethane gas.

Quench the reaction by adding 5 µl of glacial acetic acid.

Wash twice with 500 µl MeOH:H₂O (1:1). Discard the aqueous wash each time.

Dry the organic phase under N₂ stream or SpeedVac.

Reconstitute in 30 µl LC-MS grade MeOH. Transfer to LC-MS vials or 96-well plate.

Store at −80°C until analysis.

CRITICAL TMSD is toxic and generates diazomethane gas. Perform all TMSD steps in a fume hood with appropriate PPE. Open tubes briefly after 30 seconds to vent gas pressure — failure to do so risks tube rupture. Add acetic acid promptly after 10 minutes to quench. Over-methylation can modify other functional groups. Use TMSD from a freshly opened or recently opened ampoule. Check that the solution is yellow — a colourless solution indicates degraded reagent and will give incomplete methylation.

TIP If quantitative PIP analysis is required, add deuterated PIP internal standards (e.g. d5-PI(3)P or equivalent) to the MeOH in the pre-extraction step. The reconstitution volume (30 µl) is deliberately small to concentrate PIPs for detection. Discuss with the facility if signal is too low or if larger injection volumes are possible.

4.  LC-MS Analysis

| Reagent / Item | Detail / Specification |
| --- | --- |
| Methylated PIPs | Lipids Negative method (primary) and Lipids Positive method |

NOTE PIPs are typically detected in negative ion mode as [M−H]⁻ ions. Discuss analysis method selection with the facility before submission — specific column and mobile phase conditions are used for PIP chromatography.

5.  Summary

| Reagent / Item | Detail / Specification |
| --- | --- |
| Input | Cell pellet (~1 million cells) |
| Neutral lipid removal | 1 ml MeOH:CHCl₃ (2:1); sonicate 10 min; discard supernatant |
| PIP partitioning | 1000 µl MTBE/MeOH/2N HCl (24:74:2) + 250 µl 0.1N HCl |
| Derivatisation | 50 µl 2M TMSD, 10 min RT; quench with 5 µl acetic acid |
| Wash | 2 × 500 µl MeOH:H₂O (1:1) |
| Reconstitution | 30 µl LC-MS grade methanol |
| Storage | −80°C until analysis |
| Analysis mode | Negative ion mode LC-MS |